Potentiometer Analog Input with Arduino

A complete guide and step-by-step video script for building a potentiometer-controlled blinking LED using Arduino Uno

Introduction

One of the very first lessons every Arduino beginner learns is how to read an analog signal. Unlike digital signals, which are simply HIGH (5V) or LOW (0V), analog signals can vary smoothly across a range of values. A potentiometer is the perfect component to demonstrate this concept: by turning its knob, you produce a variable voltage that the Arduino can read and use to control other components — in this case, an LED.

In this project, we will connect a potentiometer to the Arduino's analog input pin (A0), and use the value it reads to control how fast an LED on pin 13 blinks. Turn the knob one way, and the LED blinks quickly. Turn it the other way, and it blinks slowly. This is a simple but powerful demonstration of how physical input can directly influence a program's behavior in real time.

We will build and simulate this project in Arduino Uno, which lets you design, wire, and test electronics entirely in the breadboard.

What You Will Learn

- How a potentiometer works as a voltage divider

- How to read analog values with analogRead()

- How analog values (0–1023) map to real-world behavior

- How to control an LED's blink rate using sensor input

- How to wire a breadboard circuit correctly and safely

- Components Required

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino Uno | 1 | Microcontroller board |
| Breadboard | 1 | For prototyping the circuit |
| Potentiometer (10kΩ) | 1 | Analog input / voltage divider |
| LED (any color) | 1 | Visual output |
| Resistor (220Ω) | 1 | Current-limiting resistor for the LED |
| Jumper wires | Several | Connections between components |
| USB cable | 1 | Power and programming |
| (Optional) Multimeter | 1 | To measure voltage/current for demonstration |

How It Works

A potentiometer has three pins:

Two outer pins — connected across the power supply (5V and GND). This creates a fixed voltage divider.

One middle pin (the wiper) — outputs a variable voltage between 0V and 5V, depending on the knob's position.

The Arduino's analog input pin (A0) reads this variable voltage and converts it into a digital number between 0 and 1023 using its built-in 10-bit Analog-to-Digital Converter (ADC):

- Knob fully one direction → wiper reads close to 0V → analogRead() returns close to 0

- Knob fully the other direction → wiper reads close to 5V → analogRead() returns close to 1023

In our program, this number is used directly as a delay value in milliseconds for blinking the LED. A small number means short delays (fast blinking); a large number means long delays (slow blinking).

Circuit Wiring

- Place the potentiometer on the breadboard.

- Connect one outer pin of the potentiometer to the breadboard's + (5V) rail.

- Connect the other outer pin to the breadboard's – (GND) rail.

- Connect the potentiometer's middle pin (wiper) to Arduino pin A0.

- Place the LED on the breadboard (long leg = anode/positive, short leg = cathode/negative).

- Connect the LED's cathode (short leg) through a 220Ω resistor to the GND rail.

- Connect the LED's anode (long leg) to Arduino pin 13.

- Connect the Arduino's 5V pin to the breadboard's + rail, and GND pin to the breadboard's – rail.

That's the entire circuit — simple, but it teaches a fundamental interaction pattern used in countless real projects (dimmers, volume knobs, speed controllers, sensors, etc.).

Code Breakdown

- pinMode(A0, INPUT); — sets A0 as an input pin (this is actually optional for analog pins, but good practice).

- pinMode(13, OUTPUT); — configures pin 13 to drive the LED.

- analogRead(A0); — reads the voltage at A0 and returns a number from 0 to 1023.

- digitalWrite(13, HIGH/LOW); — turns the LED on or off.

- delay(sensorValue); — pauses execution for a duration directly tied to the potentiometer's position.

Real-World Applications

This simple analog-read pattern is the foundation for many real projects:

- Volume knobs for audio devices

- Brightness dimmers for lighting

- Joystick and game controller inputs

- Speed control for motors and fans

- Sensor calibration (temperature, light, distance sensors, etc.)

Conclusion

This project is a small one, but it introduces one of the most important concepts in embedded electronics: turning a physical, continuous action (turning a knob) into a digital value your code can use. Once you understand this pattern, you can apply it to nearly any analog sensor — light sensors, flex sensors, soil moisture sensors, and more.